INPUT CAPACITOR SELECTION:

Input capacitor is used to average out the pulsating input current due to switching operation thus also reducing the input voltage ripple.

First Calculate Duty cycle for worst case:

DMAX=VOUT VIN MIN  ,   DMIN=VOUT VIN MAX

Select D as maximum value from the set of (DMAX, DMIN, 0.5)

Since Input capacitor rms current is usually worst at D=0.5.

After this use the following formula to calculate Input capacitor RMS current:

IIN CAP RMS=((ISWPK-IIN AV)2+∆ISW PP212)×D+(IIN AV)2×(1-D)

Where ISWPK=IOUTMAX+∆IL2, ∆IL being the inductor current ripple.

∆ISW PP=∆IL

IIN AV=D×IOUTMAX

Now, based on the input peak to peak ripple voltage requirement which can be divided into capacitor based ripple and ESR Based ripple. Based upon the capacitor ripple, Capacitor value is :

CIN MIN=D×IIN AVFs ×VIN Cap RIPPLE

And based upon the ESR ripple ∆VESR, pp, ESR maximum value for the capacitor to be selected is given by:

ESRMax=∆VESR, ppFsIpp. IN

Now Based upon the above CIN MIN value, select Capacitor based upon:

- Vbias=Vin, max

- Worst expected temperature 85-105C

- If C vs Voltage curves are not present, then keep Vbias≤ 0.5×Vrated and assume 50 to 70 %  drop in C at bias.

- For voltage rating while selecting the capacitor, for MLCC capacitors, keep Vrated≥2×Vin, max. And for polymer electrolytic capacitors, keep Vrated≥1.25-1.5×Vin, max

- For polymer and electrolytic, verify that the IC, RMS≤Iripple,rated

- Calculate PESR=IC, RMS2×ESR at Fs

Where Fs is the switching Frequency.

- Check whether,

Tcore=Tambient+∆T,

Tcore< Trated

∆T=θJA×PESR

  Near the FETs (HF loop): multiple small MLCCs (e.g., 1–3× 4.7–22 µF, 1210/1206) on the VIN–PGND hot loop, minimum loop area. These do most of the switching work.

  Bulk at VIN entry: polymer (e.g., 47–330 µF) if source is “soft” or cabling is long; otherwise may be optional for compact designs.

  Prefer many smaller MLCCs over one big for lower ESL, shared ripple, and DC-bias resilience.