Inductor Selection:

Based upon the electrical survivability-

- For the synchronous buck topology, the inductor is used as a filter to reduce the output current ripple

Lin MicroHenry=(VIN MAX-VOUT MIN)×DMAXFs×IOUT RIPPLE×106

DMAX=VOUT MAXVIN MIN

- In many cases While selecting inductor from the dataset, the agent looks at following information from the datasheet and compute:

- Isat,usable=0.7×Isat,datasheet

- Irms,usable=0.8×Irms,datasheet

- DCRhot=1.3×DCR25°C

This is because Isat is often quoted at:

- 20–30% inductance drop

- at 25 °C core temperature

Irms is usually quoted at:

- ΔT = 40 °C rise

- with ideal PCB copper

DCR is almost always:

- measured at 25 °C copper temperature

- Saturation current > Peak inductor current IL Peak=IOUTMAX+IOUT RIPPLE2

- Low DCR RL is preferred as it causes lower conduction loss

- Rms current relationship should also be satisfied: Irms,usable>Irms

ILRMS=IOUTMAX2+IOUT RIPPLE212

- Rule of thumb to select output current ripple IOUT RIPPLE to be from 20% to 40% of the max load current which is

IOUTMAX=POUTMAXVOUT MIN

Based upon Core properties and saturation physics-

The Agent computes the actual required saturation current by taking into account the nature of the load which is given by the following equation:

Isat required=Margin×IL Peak

The Margin here is a number which is an estimate based upon the load current transient demands. Typically, in case of robotics, Peak transient current demand is high and also not exactly certain. And only in such cases where the current spikes suddenly is that inductor prone to saturation. So, the agent takes the Margin to be around 1.6 times the IL Peak.

Now, the agent will look at the datasheet and see if the mentioned saturation current is greater than this required saturation current or not and if it is , only then the inductor is further inspected.

- Ferrite Cores which are lightly gapped or ungapped have high permeability, low core loss at high frequency but have a sudden and sharp saturation knee due to which inductance drops suddenly which can cause sudden spikes in current, thus increasing the risk of saturation and damage to other components. So when using this, Margin must be aggressively high.

- Ferrite cores which are gapped have lower permeability which causes energy to be stored uniformly. This causes the saturation knee to be soft and gradual. So, Inductance drops gradually. The trade-off is higher copper loss. This type of core has medium risk of saturation, so a medium margin is required for saturation current.

- Finally for distributed Air gap cores, the gaps are spread uniformly, so energy is naturally spread uniformly throughtout. Inductance degrades gracefully. Efficiency drops first and temperature rises slowly. So, these are highly preferred for robotics and transient heavy loads.

Agent first identifies core from manufacturer datasheet and extracts following information:

- % L (Inductance) drop at operating current (peak current)

- Slope of L(I)

If the knee of the curve is sharp, then saturation risk is high. It classifies risk according to following table:

| Class | Description | Risk |
| --- | --- | --- |
| A | Distributed gap, smooth | Low |
| B | Gapped ferrite | Medium |
| C | Sharp ferrite knee | High |

Agent catches the major failure mode in robotics which is magnetics saturation.

Based upon Loss Modelling and Energy Reality-

The two losses in inductors are Core loss and Copper Loss. Within the Copper loss, there are DC copper loss and AC Copper loss.

DC Conduction or DC Copper Loss (in Watts W) in the Inductor is

PDCCopper=ILRMS2×RL

RL is the DCR (DC Resistance) of the Inductor

AC Conduction or AC Copper Loss (in Watts W) in the Inductor is

PACCopper=PDCCopper×KAC

Where KAC can be estimated empirically from Vendor inductor notes using the following table:

| Condition | Effective Cu Loss Multiplier KAC |
| --- | --- |
| < 50 kHz | 1.0 |
| 50–150 kHz | 1.2 |
| 150–300 kHz | 1.4 |
| > 300 kHz | 1.6 |

The agent will reason as follows: At the selected switching frequency, AC effects in the winding significantly increase effective resistance beyond DC values. Since detailed AC resistance data is unavailable, a conservative frequency-dependent correction is applied to avoid under-estimating copper loss.

- We can even estimate Inductor Core loss PCore from the manufacturers datasheet and roughly estimate  it from steimetz equation can be used:

PCore=∆Bβ×Fswα×k

Where:

- α≈1.2 to 1.8

- β≈2.0 to 2.7

- What agent does is extract Core loss vs frequency curves, get multiple points to extract α and β. Once it gets those, then use them and one operating point to get k. This can be used to calculate exact core loss in Watts.

Based upon the comparison between core and copper loss, the agent reasons that if core loss dominates copper loss, then thermal risk is likely.

Often Vendor curves are not provided. In such cases the agent will estimate core loss stress by omitting k value and identifying frequency and ∆B dependency. The agent uses the following table to extract α and β:

| Core class | α | β | Relative loss |
| --- | --- | --- | --- |
| Ferrite | ~1.5 | ~2.5 | Low @ HF, sharp rise |
| Powdered iron | ~1.3 | ~2.0 | Higher baseline, softer |
| Sendust / Kool-Mu | ~1.3 | ~2.1 | Moderate |

And uses 
PCoreStress is proportional to (∆Bβ×Fswα)

Where ∆B is proportional to ∆I/Isat

Now the agent plugs in the values and estimates core stress and compares it to copper loss. If the ripple is high, if the switching frequency is stressing the core loss more than the copper loss, then the agent should flag this risk.

Basically, the agent will do a loss analyisis to identify which of the chosen inductors among the electrically survivable ones and the ones who won’t likely saturate are thermally safe. This way it choses and recommends the inductors which can be used in the design.

- Other factors to consider:

  Prefer molded-shielded SMD inductors for low radiated EMI in dense layouts.

  Check mechanical height, pad size, and keepout if mechanical constraints are provided by the user

- We can predict temperature rise above ambient to be given by ΔT≈Ptot​⋅Rθ​ where Ptot is the total power loss in watts approximately to be sum of PCore and PCopper.  And Rθ​ (thermal Resistance) value we can get from the datasheet.

TInductor=Tambient+∆T,

TInductor< Tlimit

Where Tlimit is defined based upon the core material used. Typicaly for   Ferrite cores and molded: Tlimit is 125C