OUTPUT CAPACITOR:

COUT MIN=IOUT RIPPLE8×Fs ×VRIPPLE MAX  (Considering Vout ripple )

COUT MIN=L×ILOADSTEP22 ×VUNDERSHOOT×(VIN MAX-VOUT MIN)×DMAX  (Considering Undershoot)

COUT MIN=L×ILOADSTEP22 ×VOVERSHOOT×VOUT MAX (Considering Overshoot)

- From the above three values, select the Max Value. Select the capacitor from the dataset.

- Among Ceramics and Electrolytic capacitors, ceramics are preferred since they have low ESR values. Once selected the Capacitor, parse the datasheet for following information Cost, ESR, ESL, lifetime.

After selecting Cout required, plug in the value in the following equations to calculate the actual voltage ripple at the output as well the initial voltage spike during the load transient.

VRIPPLE MAX=IOUT RIPPLE8×Fs ×COUT MIN+IOUT RIPPLE×ESR+VIN MAX×ESLL

VSPIKE=ILOADSTEP×ESR+di×ESLdt

Where di/dt is the load step slew rate provided by the user.

Points to be considered

- For bulk storage Electrolytic capacitors are preferred

- From the datasheet of the capacitors, check whether RMS ripple current rating of the capacitor is greater than or equal to IOUT RIPPLE12 (should be atleast 1.2 times the IOUT RIPPLE12)

- Consider capacitor derating, so assume effective capacitance[1] ≈ 0.5 × Nominal Value

- As temperature increases, the effective capacitance decreases. So consider multiplying the above calculated effective capacitance[1] by 0.8

So, now the total effective capacitance C Effective=0.8×0.5×Nominal Capacitance value from the datasheet.

- Also consider capacitor voltage rating, it should be 2×VOUT MAX since as the voltage increases, the effective capacitance decreases.

- Then also look for capacitor lifetime, we should maximize it.

- Finally consider loop stability also, since we already know the inductance value L from our calculations, so the double pole frequency is FLC=12πL×C Effective , Ensure this is 1/5 to 1/10 of the cross over frequency which itself is usually 1/10-1/20th of the Switching Frequency Fs

Basically, Select the output capacitor so that it meets ripple and transient specs while ensuring loop stability, survives the RMS ripple current and thermal life requirements, and keeps its effective capacitance after bias and temperature derating